| Washing laboratory glassware using chromic acid cleaning solution TITLE OF PERSON WHO DOES JOB: SUPERVISOR: ANALYSIS BY: JOB TASK: DATE: |  |  |  |
| --- | --- | --- | --- |
| Various trained personnel | Jack Brown / Lindy Seip |  |  |
| PSES REVIEWED BY: SECTION: DEPARTMENT: |  |  |  |
| Eye protection, acid-resistant gloves, lab coat, fume hood REQUIRED AND/OR RECOMMENDED PERSONAL PROTECTIVE EQUIPMENT: APPROVED BY: |  |  |  |
| SEQUENCE OF BASIC JOB STEPS Beware of being too detailed, record only the information needed to describe each job action. Rule of thumb, no more than 10 steps/task being evaluated | POTENTIAL ACCIDENTS OR HAZARDS Hazard Classification Categories: Stuck By/Against, Caught In/Between, Slip, Trip, or Fall, Overexertion, Ergonomic (Awkward Postures, Excessive Force, Vibration, Repetitive Motion) |  | RECOMMENDED SAFE JOB PROCEDURE Hazard Control Categories: Engineer Out (New Way to Do, Change Physical Conditions or Work Procedures, Adjust/Modify/Replace Work Station Components/Tools, Decrease Performance Frequency), Personal Protective Equipment (PPE), Training, Improve Housekeeping |
| Using a glass pipet, transfer cleaning solution from storage bottle to item to be cleaned. | Avoid spilling or splashing cleaning solution. Handle glassware carefully to prevent breaking. |  | Wear acid-resistant gloves, safety glasses and lab coat. Have all equipment in the fume head and provide adequate room to work. Do not return unused cleaning solution to original bottle. |
| Thoroughly clean glassware item | Handle glassware carefully to avoid breaking |  | Gently swirl glassware item containing cleaning solution and/or use brush to reach all interior surfaces. |

| SEQUENCE OF BASIC JOB STEPS Beware of being too detailed, record only the information needed to describe each job action. Rule of thumb, no more than 10 steps/task being evaluated | POTENTIAL ACCIDENTS OR HAZARDS Hazard Classification Categories: Stuck By/Against, Caught In/Between, Slip, Trip, or Fall, Overexertion, Ergonomic (Awkward Postures, Excessive Force, Vibration, Repetitive Motion) | RECOMMENDED SAFE JOB PROCEDURE Hazard Control Categories: Engineer Out (New Way to Do, Change Physical Conditions or Work Procedures, Adjust/Modify/Replace Work Station Components/Tools, Decrease Performance Frequency), Personal Protective Equipment (PPE), Training, Improve Housekeeping |
| --- | --- | --- |
| Pour cleaning solution from glassware item into waste beaker | Avoid spilling or splashing cleaning solution. | Have large glass beaker available in fume hood to collect waste solution. |
| Rinse glassware with distilled water | Avoid spilling or splashing cleaning solution | Use adequate rinse water to dilute acid residue in glassware. Collect waste in glass beaker and retain for proper disposal. |
| Rinse very well with additional distilled water in lab sink and allow to dry on rack. | Handle glassware carefully |  |
| Rinse all cleaning equipment with tap water. Bag and properly discard disposables | Handle all glassware with care | Carefully transfer waste solution to glass waste storage bottle. Label waste solution and record on lab waste log. |
| SEQUENCE OF BASIC JOB STEPS Beware of being too detailed, record only the information needed to describe each job action. Rule of thumb, no more than 10 steps/task being evaluated | POTENTIAL ACCIDENTS OR HAZARDS Hazard Classification Categories: Stuck By/Against, Caught In/Between, Slip, Trip, or Fall, Overexertion, Ergonomic (Awkward Postures, Excessive Force, Vibration, Repetitive Motion) | RECOMMENDED SAFE JOB PROCEDURE Hazard Control Categories: Engineer Out (New Way to Do, Change Physical Conditions or Work Procedures, Adjust/Modify/Replace Work Station Components/Tools, Decrease Performance Frequency), Personal Protective Equipment (PPE), Training, Improve Housekeeping |