| Planting potatoes with Kimberly research planter TITLE OF PERSON WHO DOES JOB: SUPERVISOR: ANALYSIS BY: JOB TASK: DATE: |  |  | April 27, 2009 |
| --- | --- | --- | --- |
| Various trained personnel | Nora Olsen |  | Nora Olsen/Richard Hayes |
| PSES REVIEWED BY: SECTION: DEPARTMENT: |  | Kimberly R & E Center | Don Morishita, George Porter, EHS |
| Eyewear, tight-fitting work gloves, hat, sunscreen and nuisance dust mask (if appropriate) REQUIRED AND/OR RECOMMENDED PERSONAL PROTECTIVE EQUIPMENT: APPROVED BY: |  |  |  |
| SEQUENCE OF BASIC JOB STEPS Beware of being too detailed, record only the information needed to describe each job action. Rule of thumb, no more than 10 steps/task being evaluated | POTENTIAL ACCIDENTS OR HAZARDS Hazard Classification Categories: Stuck By/Against, Caught In/Between, Slip, Trip, or Fall, Overexertion, Ergonomic (Awkward Postures, Excessive Force, Vibration, Repetitive Motion) |  | RECOMMENDED SAFE JOB PROCEDURE Hazard Control Categories: Engineer Out (New Way to Do, Change Physical Conditions or Work Procedures, Adjust/Modify/Replace Work Station Components/Tools, Decrease Performance Frequency), Personal Protective Equipment (PPE), Training, Improve Housekeeping |
| Mount/Dismount from planter. | Falling off planter; Misstep on uneven ground or parts of planter. Trying to dismount when moving. |  | Training on where to step when mounting and dismounting and not to move when planter moving. Face machine and maintain three points of contact when mounting or dismounting machine. |
| Operators are seated and filling segments of a revolving plate with seed potatoes. | Finger and clothing inch hazard between plate and base. Fingers in drop hole could be pinched. |  | Training on where to place and not place hands/fingers. Reminder label applied to areas of concern. Personnel to not wear loose clothing. |
| Sometimes a hopper automatically feeds via chained cup mechanism to the revolving plate. There is a guard tube around the chain/cup mechanism to keep fingers out. | The point of discharge can become clogged with seed potatoes. When this occurs, the seed must be manually cleared. A wooden stake is provided and must be used for this purpose. Hands must be kept clear of this area. |  | Do not operate unless chain/cup guard tube is in place. Training on keeping fingers clear of discharge area and utilizing wooden stick to clear clogged area or stop tractor/planter immediately. Reminder labels applied to area of concern. |

| SEQUENCE OF BASIC JOB STEPS Beware of being too detailed, record only the information needed to describe each job action. Rule of thumb, no more than 10 steps/task being evaluated | POTENTIAL ACCIDENTS OR HAZARDS Hazard Classification Categories: Stuck By/Against, Caught In/Between, Slip, Trip, or Fall, Overexertion, Ergonomic (Awkward Postures, Excessive Force, Vibration, Repetitive Motion) | RECOMMENDED SAFE JOB PROCEDURE Hazard Control Categories: Engineer Out (New Way to Do, Change Physical Conditions or Work Procedures, Adjust/Modify/Replace Work Station Components/Tools, Decrease Performance Frequency), Personal Protective Equipment (PPE), Training, Improve Housekeeping |
| --- | --- | --- |
|  |  | . |
| SEQUENCE OF BASIC JOB STEPS Beware of being too detailed, record only the information needed to describe each job action. Rule of thumb, no more than 10 steps/task being evaluated | POTENTIAL ACCIDENTS OR HAZARDS Hazard Classification Categories: Stuck By/Against, Caught In/Between, Slip, Trip, or Fall, Overexertion, Ergonomic (Awkward Postures, Excessive Force, Vibration, Repetitive Motion) | RECOMMENDED SAFE JOB PROCEDURE Hazard Control Categories: Engineer Out (New Way to Do, Change Physical Conditions or Work Procedures, Adjust/Modify/Replace Work Station Components/Tools, Decrease Performance Frequency), Personal Protective Equipment (PPE), Training, Improve Housekeeping |