### Simple Lockout/Tagout Control Procedure – SLT

The SLT procedure offers hazardous energy protection which must be implemented when the exposure to the hazards of unexpected energization or start up of the equipment cannot be controlled by the unplugging of the equipment from the energy source and/or the plug will not remain under the exclusive control of the employee performing the servicing or maintenance.

Simple lockout/tagout may only be used when all of the following conditions exist:

- the machine or equipment has only ONE hazardous energy source that can be readily identified or isolated

- there is no potential for stored or residual energy or reaccumulation of stored energy after shutdown which could endanger employees

- the isolation and locking out of the energy source will completely deenergize and deactivate the machine or equipment

- the machine or equipment is isolated from that energy source and locked out during servicing or maintenance

- a single lockout device will achieve a locked-out condition and is used to maintain a locked-out condition while servicing/maintenance is being performed

- the lockout device is under the exclusive control of the authorized employee(s) performing the servicing or maintenance

- the servicing or maintenance does not create hazards for other employees

- there have been no accidents involving the unexpected activation or reenergization of the machine or equipment during servicing or maintenance while using this procedure

- the employee has not been instructed to follow more restrictive procedures and there are no signs or labels posted indicating the need to follow more restrictive measures

The following list contains some examples of types of machines, equipment and systems that have been evaluated by qualified personnel and which have been determined to fall within the guidelines of the Simple Lockout/Tagout (SLT) procedure.

(1/3/05)

Simple Lockout/Tagout Control Procedure

Machine/Equipment List

The machines and equipment listed below have been evaluated by qualified personnel and it has been determined that they may be serviced or maintained using the Simple Lockout/Tagout Control Procedure described in the 														                                                                                                                    (Department/Division)

lockout/tagout program, providing the employee performing the service or maintenance is trained and authorized according to the program=s requirements.